Using UTM/MGRS

I have provided a number of maps from Manitoba and north western Ontario.  Locate the following points on the 1:50 000 scale maps using UTM co-ordinates. Remember to check the datum for your map.

| Map Identifier | Datum | Find Point | Point | UTM for point | UTM |
| --- | --- | --- | --- | --- | --- |
| Pickerel Lake, 52 B/11 |  | 15U E619500 N5384500 |  | Pine Portage in Pine Portage Bay at east end of Pickerel Lake |  |
| Caddy Lake, 52 E14 |  | 15U E345200 N5539660 |  | Tunnel under CP tracks at North end of Caddy Lake |  |
| Aikens Lake (Color), 52 M/3 |  | 15U E334240 N5657275 |  | Last set of Rapids on the Gammon River east of Aikens Lake |  |
| Aikens lake (B/W), 52 M/3 |  | 15U E334250 N5657050 |  | Last set of Rapids on the Gammon River east of Aikens Lake |  |
| Dryberry Lake |  | 15U E434550 N5487950 |  | Pictographs on cliff in channel at north end of Teggau Lake |  |

If you are ‘GeoCaching’ the most common co-ordinate format that is used is Latitude/Longitude in the format HDD° MM.MMM’ (H represents the hemisphere - N/S, E/W).    Unfortunately measuring distances can be difficult when working with Lat/Long co-ordinates. When working with Canadian topographic maps I generally recommend using UTM or MGRS measurements because distances are easy to measure with a simple metric ruler.  MGRS (Military Grid Reference System) is just a special format for describing UTM co-ordinates in an abbreviated format.

When reading off of Canadian Topographic maps UTM units (meters) are the light blue numbers running along the sides of the map.  When reporting UTM or MGRS numbers they are typically listed as easting then northing numbers.  I find it easy to remember as ‘enter the door take the elevator’.

The blue numbers and lines on 1:50,000 maps are shown every 1000meters (2cm=1000m).  This makes direct measurements with a ruler or even an approximation by ‘eye’ very simple.  The full UTM co-ordinates are listed at each corner with abbreviated numbers (matching the MGRS 100 000m square reference) listed at every interval.  The 100 000 meter grid numbers in the corner UTM measurements are also enlarged.  Since the MGRS grid spans 100km it may not be necessary to provide the two letter prefix but it is a good idea.

Note when using MGRS an even number of numbers are always listed; more numbers provide greater precision. Typically two character grid identification should also be listed prior to the numbers. This identification is used to identify the specific 100 000m square within the particular UTM zone

UTM:		633827m E  5518867m N

MGRS:  	338188		(6 characters indicates 100m)

33821886		(8 characters indicates 10m)

3382718867 	(10 characters indicates 1m)